Haptic Feedback Devices: Switching from Bluetooth Classic to ESP-NOW

Adding a Master device and a Wi-Fi path for the Meta Quest 3

Progress Report

1. The Task

The haptic feedback devices currently receive their commands from the computer using Bluetooth Classic. There was no separate master device — the computer talked to each haptic device directly.

The tasks given were:

1.  Rewrite the system so the devices communicate using ESP-NOW instead of Bluetooth Classic.

2.  Create a Master device that receives data from the computer over a serial (USB) connection and forwards it to the haptic devices using ESP-NOW.

3.  Investigate whether the Meta Quest 3 VR headset can talk to a Master device. The Quest cannot use ESP-NOW directly, so it would need a Master. The idea was to connect the Quest to the Master by USB cable and check if a Unity app on the Quest can communicate over a serial connection.

2. Understanding the Original Code

The supervisor's original code runs on one haptic device and does two things. First, it receives a command from the computer over Bluetooth Classic. Second, it uses five values from that command to drive a motor that produces the physical feedback the user feels.

The five values in every command are:

- spring impact — the main pushing force

- damping — resistance that depends on how fast the motor turns

- vibration amplitude — how strong the vibration is

- vibration frequency — how fast the vibration is

- brake — turns a brake on or off

The command arrives as a simple line of text with the five values separated by semicolons, for example:

1.5 ; 0.3 ; 0.8 ; 40 ; 1

Only the way the command ARRIVES needed to change (from Bluetooth to ESP-NOW). The motor part of the code stays exactly the same, because the five values arrive in the same form either way.

3. What I Did — Part 1: ESP-NOW with a Master

I built the new system with one Master device and four haptic devices (slaves). The computer sends a command to the Master over USB. The command includes a device number, so the Master knows which device it is for. The Master then sends the five values to that specific device using ESP-NOW.

The command format

A device number was added at the front of the command, so the Master knows which device to send to:

deviceID ; spring ; damping ; vibrationAmplitude ; vibrationFrequency ; brake

For example, this command tells Device 2 what to do:

2 ; 1.5 ; 0.3 ; 0.8 ; 40 ; 1

The setup

Computer --USB Serial--> Master --ESP-NOW--> Haptic Device (correct one)

Because I do not have a real motor, the haptic device does not move a motor yet. Instead it prints the values it received, to prove the correct data arrived. Later, the supervisor's motor code can be placed exactly where the print is — the five values are already there, ready to use.

4. Result — Part 1 (USB Serial)

I sent commands from the computer to all four devices. The Master correctly forwarded each one:

Command> 1;1.5;0.3;0.8;40;1   ->  Sent to Device 1
Command> 2;2.0;0.5;1.0;50;0   ->  Sent to Device 2
Command> 3;0.5;0.1;0.3;30;1   ->  Sent to Device 3
Command> 4;1.0;0.2;0.5;35;0   ->  Sent to Device 4

Device 1 was checked directly and confirmed it received exactly the values that were sent:

Device 1 received: spring=1.50 damping=0.30 vib_amp=0.80 vib_freq=40.00 brake=1

| Check | Result |
| --- | --- |
| Computer sends command to Master over USB | Working |
| Master sends to the correct device | Working |
| Device receives the correct values | Working |
| Each device responds only to its own number | Working |

Conclusion: The ESP-NOW system with a Master works. This replaces Bluetooth Classic successfully.

5. What I Did — Part 2: The Meta Quest 3

The second task was to check whether the Quest 3 can talk to a Master device, and whether a USB cable connection would work for this.

Can the Quest use ESP-NOW directly?

No. ESP-NOW only works between ESP32 chips. The Quest 3 is a VR headset and has no ESP32 inside, so it cannot join an ESP-NOW network. This is why it needs a Master device to act as a bridge.

Can the Quest connect to the Master by USB cable?

I investigated this and found that a USB cable connection does not work, for two reasons:

- Host and device mismatch: A USB connection needs one side to be the 'host' (in charge) and the other to be the 'device'. The Quest 3 acts as a device, and the ESP32 also acts as a device. Two devices cannot communicate over USB because neither one is the host.

- No serial support: Even if the connection worked, the Quest's operating system (Horizon OS) does not provide a serial/COM port that a microcontroller can use. There is also no driver for the ESP32's USB chip.

So connecting the Quest 3 to a Master by USB cable is not a workable option.

The solution that works: Wi-Fi

The working method is Wi-Fi. The Quest 3 can send data over Wi-Fi to the Master, and the Master forwards it to the haptic devices over ESP-NOW. I built and tested this Wi-Fi path.

Because I do not have a Quest 3, I used the computer to send the Wi-Fi data instead. To the Master, a Wi-Fi message from the computer looks exactly the same as one from the Quest. So this proves the Master side works. When a real Quest is available, a small Unity app on the Quest sends the same Wi-Fi message.

The setup

Quest 3 (or computer) --Wi-Fi--> Master --ESP-NOW--> Haptic Device

6. Result — Part 2 (Wi-Fi)

A problem I found and fixed

At first the Master received the Wi-Fi command, but the device received nothing. The reason was a Wi-Fi channel mismatch. When the Master joins a Wi-Fi router, the router puts it on a certain channel (channel 6 in this test). But ESP-NOW needs the Master and the devices to be on the same channel. The devices were still on channel 1, so the messages could not reach them.

I fixed this by locking all the haptic devices to the same channel as the router (channel 6). After the fix, the Wi-Fi path worked perfectly.

Test result

After the fix, a command sent over Wi-Fi reached the device correctly:

Master:   UDP received: 1;1.5;0.3;0.8;40;1
Master:   Sent to Device 1
Device 1: Locked to Wi-Fi channel: 6
Device 1: received: spring=1.50 damping=0.30 vib_amp=0.80 vib_freq=40.00 brake=1

| Check | Result |
| --- | --- |
| Master connects to Wi-Fi | Working |
| Master receives the Wi-Fi command | Working |
| Channel mismatch problem | Found and fixed |
| Device receives correct values over Wi-Fi | Working |

Conclusion: The Wi-Fi path works. This confirms the Quest 3 will be able to control the devices this way once a Quest is available for the final test.

7. Summary of Findings

| Method | Works? | Reason |
| --- | --- | --- |
| Quest to devices by ESP-NOW directly | No | Quest has no ESP32 inside |
| Quest to Master by USB cable | No | Host/device mismatch; no driver |
| Quest to Master by Wi-Fi | Yes | Tested and working |
| Computer to Master by USB | Yes | Tested and working |
| Computer to Master by Wi-Fi | Yes | Tested and working |

8. Current Status

| Task | Status |
| --- | --- |
| Convert devices from Bluetooth Classic to ESP-NOW | Done and tested |
| Master: computer over USB, forward by ESP-NOW | Done and tested |
| Master: Wi-Fi in, forward by ESP-NOW | Done and tested |
| Send correct values to a specific device | Done and tested |
| Investigate Quest 3 USB connection | Done - does not work |
| Wi-Fi path for the Quest 3 | Done (tested with a PC) |
| Add real motor code to the devices | To be done by supervisor |
| Final test with an actual Quest 3 | Waiting for a Quest 3 |

9. Next Steps

- Add the motor control code to the haptic devices. In the test code each device prints the received values. The existing motor code can be placed where the print is — the five values are already available.

- Confirm all four devices individually when a suitable USB cable is available. The code is identical for all four, only the device number differs, so this is a formality.

- When a Quest 3 is available, add a small Unity app that sends the command over Wi-Fi, and run the final test.

Important note about the Wi-Fi channel

Everything is currently set to Wi-Fi channel 6, because that is the channel of the test router (TP-LINK). If a different router is used, its channel may be different. In that case, check the channel the Master prints when it starts, and set the same channel number in the devices.